# 编译部分：

- 创建新workspace，先编译以下功能包

https://github.com/UniversalRobots/Universal_Robots_ROS2_Driver/tree/humble?tab=readme-ov-file （这个是Universal_Robots_ROS2_Driver的github仓库，他会让你补充Universal_Robots_Client_Library，里面还会有https://docs.ros.org/en/ros2_packages/humble/api/ur_robot_driver/doc/installation/installation.html 这个网址告诉你如何编译，核心是colcon build --cmake-args -DCMAKE_BUILD_TYPE=Release

）

- 在刚刚的workspace加入

然后在cartesian_controllers官方github仓库：

https://github.com/fzi-forschungszentrum-informatik/cartesian_controllers/tree/ros2

找到编译方法，然后进入workspace继续编译

- 创建新工作空间，克隆moveit2并编译，官方github仓库有教程，记得选好branch

https://github.com/moveit/moveit2?tab=readme-ov-file

# 运行部分

- 窗口1运行：

ros2 launch ur_simulation_gz ur_sim_control.launch.py ur_type:=ur5e

- 窗口2运行：

- source ws_moveit2/install/setup.bash

ros2 launch ur_moveit_config ur_moveit.launch.py ur_type:=ur5e use_sim_time:=true launch_rviz:=true launch_servo:=true 【注意：仿真时在launch文件加入了使用sim time】

- 窗口3运行

ros2 run rqt_controller_manager rqt_controller_manager,

然后deactivate一开始开启的joint controller，然后activate forward_position_controller(若末端位控则activate cartesian_controller)

- 窗口4（启动servo，Humble的Servo需要start服务）

先ros2 service list | grep start_servo，应该看到/servo_node/start_servo

然后调用ros2 service call /servo_node/start_servo std_srvs/srv/Trigger {}

然后检查状态topic，data=0则为正常：ros2 topic echo /servo_node/status

- 找到输入 topic 并确认订阅者（servo_node会订阅）

ros2 topic info /servo_node/delta_twist_cmds -v

- 发 TwistStamped 测试 EE 速度控制

ros2 topic pub -r 100 /servo_node/delta_twist_cmds geometry_msgs/msg/TwistStamped "{

header: {stamp: 'now', frame_id: 'tool0'},

twist: {linear: {x: 0.0, y: 0.0, z: 0.1}, angular: {x: 0.0, y: 0.0, z: 0.0}}

}"

- 若用cartesian_controller，则用posestamp测试位置控制

ros2 topic pub -1 /cartesian_motion_controller/target_frame geometry_msgs/msg/PoseStamped "{

header: {frame_id: 'base_link'},

pose: {

position: {x: 0.001, y: 0.233, z: 1.079},

orientation: {x: -0.707, y: 0.001, z: 0.001, w: 0.707}

}

}"

Control Physical Robot

Quick startup:

bash ur5e_startup.sh

-------------------------------------------------------------------------------------------------------------

Step by step startup:

ros2 launch ur_robot_driver ur_control.launch.py ur_type:=ur5e robot_ip:=192.168.10.102

- source ws_moveit2/install/setup.bash

ros2 launch ur_moveit_config ur_moveit.launch.py ur_type:=ur5e use_sim_time:=false launch_rviz:=true launch_servo:=true

ros2 run rqt_controller_manager rqt_controller_manager

Then, close joint trajectory controller

Then, activate forward_position_controller

ros2 service list | grep start_servo

然后调用ros2 service call /servo_node/start_servo std_srvs/srv/Trigger {}

然后检查状态topic，data=0则为正常：ros2 topic echo /servo_node/status

Based on EE coordinate:

ros2 topic pub -r 100 /servo_node/delta_twist_cmds geometry_msgs/msg/TwistStamped "{

header: {stamp: 'now', frame_id: 'tool0'},

twist: {linear: {x: 0.0, y: 0.0, z: 0.1}, angular: {x: 0.0, y: 0.0, z: 0.0}}

}"

Based on Base coordinate:

ros2 topic pub -r 100 /servo_node/delta_twist_cmds geometry_msgs/msg/TwistStamped "{

header: {stamp: 'now', frame_id: 'base'},

twist: {linear: {x: 0.0, y: 0.0, z: 0.1}, angular: {x: 0.0, y: 0.0, z: 0.0}}

}"

-------------------------------------------------------------------------------------------------------

Orbbec Gemini2L RGBD Camera

ros2 launch orbbec_camera gemini2L.launch.py depth_format:=Y14

Solution:

1)Find “~/orbbect_ws/src/OrbbecSDK_ROS2/orbbec_camera/launch/gemini2L.launch.py”

2) Modify ‘depth_format’ value from “Y16” to “Y14”

DeclareLaunchArgument("depth_format", default_value="Y14"),

3) Complie

cd orbbect_ws

colcon build --event-handlers  console_direct+  --cmake-args  -DCMAKE_BUILD_TYPE=Release

4) Run ‘ros2 launch orbbec_camera gemini2L.launch.py’

5) Open rviz2 to display the RGB image / depth image / Point cloud

Mid-360

Installation:

1) Follow github readme installation steps to install Livox-SDK2 and livox_ros_driver2

2) Run

source ~/ws_livox/install/setup.bash

ros2 launch livox_ros_driver2 rviz_MID360_launch.py

ERROR 1:

Normally, it will print an ERROR messages in the terminal:

“terminate called after throwing an instance of 'class_loader::LibraryLoadException'

what():  Could not load library dlopen error: liblivox_lidar_sdk_shared.so: cannot open shared object file: No such file or directory, at ./src/shared_library.c:99

[ros2run]: Aborted”

Solution：

Add one line in .bashrc file

1) Open a terminal, run the following command,

export LD_LIBRARY_PATH=/usr/local/lib

2) Rerun the command

ros2 launch livox_ros_driver2 rviz_MID360_launch.py

ERROR 2:

rob@robnuc:~/ws_livox$ ros2 launch livox_ros_driver2 rviz_MID360_launch.py

[INFO] [launch]: All log files can be found below /home/rob/.ros/log/2026-01-12-18-00-57-848143-robnuc-6277

[INFO] [launch]: Default logging verbosity is set to INFO

[INFO] [livox_ros_driver2_node-1]: process started with pid [6278]

[INFO] [rviz2-2]: process started with pid [6280]

[livox_ros_driver2_node-1] [INFO] [1768212057.902159952] [livox_lidar_publisher]: Livox Ros Driver2 Version: 1.2.4

[livox_ros_driver2_node-1] [INFO] [1768212057.902493986] [livox_lidar_publisher]: Data Source is raw lidar.

[livox_ros_driver2_node-1] [INFO] [1768212057.902514468] [livox_lidar_publisher]: Config file : /home/rob/ws_livox/install/livox_ros_driver2/share/livox_ros_driver2/launch_ROS2/../config/MID360_config.json

[livox_ros_driver2_node-1] LdsLidar *GetInstance

[livox_ros_driver2_node-1] config lidar type: 8

[livox_ros_driver2_node-1] successfully parse base config, counts: 1

[livox_ros_driver2_node-1] bind failed

[livox_ros_driver2_node-1] Failed to init livox lidar sdk.

[livox_ros_driver2_node-1] [ERROR] [1768212057.904702665] [livox_lidar_publisher]: Init lds lidar fail!

[rviz2-2] [INFO] [1768212058.174511057] [rviz2]: Stereo is NOT SUPPORTED

[rviz2-2] [INFO] [1768212058.174582048] [rviz2]: OpenGl version: 4.6 (GLSL 4.6)

[rviz2-2] [INFO] [1768212058.187690039] [rviz2]: Stereo is NOT SUPPORTED

Solution:

Configure the ip address between Mid360 and Computer

1) Open /home/rob/ws_livox/src/livox_ros_driver2/config/MID360_config.json

2) Modify the ip address of host and lidar

Host ip: our computer static IP address

Lidar ip: 192.168.1.1xx (xx is the last two number of serial number)

{

"lidar_summary_info" : {

"lidar_type": 8

},

"MID360": {

"lidar_net_info" : {

"cmd_data_port": 56100,

"push_msg_port": 56200,

"point_data_port": 56300,

"imu_data_port": 56400,

"log_data_port": 56500

},

"host_net_info" : {

"cmd_data_ip" : "192.168.1.5",

"cmd_data_port": 56101,

"push_msg_ip": "192.168.1.5",

"push_msg_port": 56201,

"point_data_ip": "192.168.1.5",

"point_data_port": 56301,

"imu_data_ip" : "192.168.1.5",

"imu_data_port": 56401,

"log_data_ip" : "",

"log_data_port": 56501

}

},

"lidar_configs" : [

{

"ip" : "192.168.1.135",

"pcl_data_type" : 1,

"pattern_mode" : 0,

"extrinsic_parameter" : {

"roll": 0.0,

"pitch": 0.0,

"yaw": 0.0,

"x": 0,

"y": 0,

"z": 0

}

}

]

}

3) Compile

rob@robnuc:~/ws_livox$ ./src/livox_ros_driver2/build.sh humble

4)Run

ros2 launch livox_ros_driver2 rviz_MID360_launch.py

Gripper

source ~/handControl2_ws/install/setup.bash

ros2 launch hand_control hand_control.launch.py port_name:=/dev/ttyUSB1 baudrate:=115200

Control:

ros2 run hand_control talker Open 1

ros2 run hand_control talker Close 1

Joystick Control

Run the below node in the home path:

source ~/joystickTele_ws/install/setup.bash

ros2 run joy_servo_control logitech_f710_servo_twist --device /dev/input/js1

Github key (Jiajie):

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